IMPACT OF MECHANIZATION ON LOWLAND RICE PRODUCTION IN NIGERIA

Faleye T1., D. James1, O. T. Dada-Joel1 , Y. S. Ademiluyi1 and T. Wakatsuki2

1National Centre for Agricultural Mechanization (NCAM). P.M.B. 1525, Ilorin, Kwara State, Nigeria.

2Faculty of Agriculture, Kinki University, Nara 29201, Japan

Corresponding author: Faleye T., M. Eng., research field: Tillage and Agricultural Machinery System Design and Testing. E-mail: topeleye@yahoo.com.

Abstract

The study examined the impact of using power tiller as a means of mechanizing lowland rice production in Nigeria. The study was carried out in Bida area, Niger state, where the sawah rice production was disseminated by Watershed Initiative in Nigeria (WIN). And in Ajase Ipo of Kwara State, some of the parameters assessed during the study included average speed of operation, average wheel slip/travel reduction, average draught of implement, and fuel consumption. The cost of operation and yield over five years of usage at Niger State and two years in Kwara State was determined and it was therefore concluded that the power tiller is the most appropriate field machinery for tillage operations such as puddling, leveling e.t.c. on lowland rice production in Nigeria.

Key words: Mechanization, Rice production and power tiller.

- INTRODUCTION

Agricultural mechanization is the application of mechanical technology and increased power to agriculture, largely as a means to enhance the productivity of human labor and often to achieve results well beyond the capacity of human labor with optimum yield at minimal cost. This includes the use of tractors of various types as well as animal-powered and human-powered implements and tools, and internal combustion engines, electric motors, solar power and other methods of energy conversion.

Mechanization also includes irrigation systems, food processing and related technologies and equipment. Mechanization is not an “all or nothing” process. Levels and types of improved mechanical technologies need to be appropriate, that is, compatible with local, agronomic, socio-economic, environmental and industrial conditions.

Rice has become an important strategic and daily staple food crop in Nigeria. The potential land area for rice production in Nigeria is between 4.6 million and 4.9 million ha. Out of this, only about 1.7 million ha or 35 percent of the available land area is presently cropped to rice [7].

The main production ecologies for rice in Nigeria are rain fed lowland; rain fed upland, irrigated lowland, deep water / floating and mangrove. Of these, rainfed lowland rice has the largest share of the rice area (50%) and rice production as shown in Table 1. New high-yielding lowland varieties of the NERICA are undergoing agronomic evaluation in Nigeria and several other countries.

Small-scale farmers with farm holdings of less than 1 ha cultivate most of the rice produced in Nigeria. However, rice productivity and production at the farm level are constrained by several factors. These constraints include insufficient appropriate technologies, biotic factors, poor supply of inputs, ineffective farmer organizations and groups, low yield and poor milling quality of local rice varieties, poor marketing arrangements, inconsistent agricultural input and rice trade policies, poor extension systems and environmental constraints. These environmental constraints include poor drainage and iron toxicity in undeveloped lowland swamps, poor maintenance of developed lowland swamps, drought, deficiencies of N and P, insufficient rain for rainfed rice production, and poor soil management practices. The size of the average family holding in many developing countries is of the order 2 to 5 hectares, with 3 hectares as an appropriate average figure. Such a holding would therefore require a power input of 1.2 kW to provide the level suggested [2]. It is therefore the opinion of many that due to the economic level of majority of farmers in developing countries like Nigeria, in transforming from the presently predominant hand- tool technology to a full blown large scale engine power technology, there has to be an appropriate intermediate technology. In the past this has been viewed as the animal draft technology. However, the introduction of two-wheel tractors (power-tillers) in many countries is proving to be a better and more appropriate intermediate technology. In the past five to six years, there has been an influx of two- wheel tractors into Nigeria and the demand has particularly been increasing in rainfed lowland areas (Table 1) where its use for the cultivation of low land rice is increasingly becoming popular.

The objectives of this study are to (1) Determine the profitability of using power tiller in rice production. (2) Determine the impact of using the power tiller as a means of mechanizing rice production.

2	METHODOLOGY

2.1 Description of the machine used: The power-tiller or walking tractor, as it is sometimes called is a single-axle (two-wheel) tractor. This particular one is of Indian make and the model is VST-SHAKTI 130 DI with 10 kW (13 hp) rated power, diesel engine of 2400 rpm rated crankshaft speed. The engine is single cylinder horizontal 4 strokes, water cooled and hand-cranking type. The driving wheels are of two types: the pneumatic type for normal traction and the steel or cage wheel for wet puddling.

2.2 Field layout, Location and operations: The operation of the machine was carried out in wet puddling on two different rice field located at Shaba-Maliki and Ejeti village near Bida on a clayey loamy, sandy soil,. Bida is 137 m above sea level and lies on longitude 6°01’E and latitude 9°06’N in Niger State of Nigeria. While Ajase Ipo of Kwara State is at an altitude of 370 m above sea level and lies on longitude 2030’E and latitude 7045N both under the Guinea savannah ecology of Nigeria. The 600 mm tine cultivator was attached to the power tiller and it was used for puddling of the field before the transplanting of the rice was done. Fashola et al3 have a detailed description of the sawah system on farmers’ fields. Some of the parameters assessed during the field test included average speed of operation, average wheel slip/travel reduction, average draught of implement and fuel consumption. The soil properties monitored included soil moisture content, bulk density, porosity, penetrometer resistance/cone index and shear strength. The core technique was used in obtaining samples for bulk density measurement, soil penetrometer and shear vane readings were determine in situ. Soil samples were obtained at various depths of 7 cm intervals, soil laboratory tests were all performed using standard procedures [5].

2.3 Cost determination: Cost calculations for mechanized farm operations are almost similar everywhere. Usually there are basic assumptions and few other adjustments are made to suit the particular needs and locality. For example, while in some countries taxes are paid on agricultural machines and implements, in Nigeria they are tax-free.

Costing of mechanized operation can be done either on an hourly basis or a hectare basis. The former is only meaningful when accurate records of the time (machine hours) are available. In Nigeria, 34.7% of the operator’s time was spent in doing unproductive work 31.2% of these representing avoidable delays [1]. It would be unfair and inaccurate to add this idling time to the overall time for carrying out an operation. Moreover, the farmer is more interested to know how much it will cost him to farm a unit area so as to compare it with the yield per area.

Generally, Machinery costs fits into two broad categories:

- Fix costs include:

The factors involved in the calculation of operating costs of farm machinery can be grouped under two main titles – “fixed and variable” factor or costs. Fixed factors include the factors whose cost must always be taken into account whether the machinery is in operation or not. They are related to machine ownership and represent a form of financial discipline, to make sure that the business does in-fact pay-off capital investment within a reasonable period. They are depreciation (on tractor and on implement), interest on investment, taxes, insurance and shelter. Many ways have been put forward for calculating depreciation. These include the inventory method, the straight line method, sum of the years’ digits method and the sinking fund method [5]. After reviewing these methods, it was found that, for the Nigerian condition, the straight line method was most appropriate. The equation used was: Depreciation = Initial cost of machine/ Expected total hours of use.

- Variable costs include:

Variable factors are the remaining operating costs that depend on particular operations and are directly proportional to the machinery usage. They include labour (operator and mechanic), spares, fuel and lubricants.

Labour – The usual daily rate for skilled workers like operators and mechanics increases from N500 in 2002 to N700 in 2006 per day while it was N 1, 000 to N 1, 200 per day in 2009 to 2010, but it is very uncommon to employ mechanics on a daily wage basis; instead they are paid whenever their services are needed in a case of sudden breakdown.

Fuel– The variables that have direct influence on the fuel consumption for any operation are speed of operation and soil properties (texture and moisture) and skill of operator. Fuel cost per ha can be given as F = fx pC 									    (1)

Where F = fuel cost/hectare (ha), f = fuel consumption rate (l/h), p = cost of fuel (l), c = rate performance of machine or machine capacity (ha/l).

The operating cost of power tiller was determined based on its annual use for a period of five years usage. The cost per year was calculated by the relation:

C = FCP100+H(R&M)104+ L+O+F                                                                 		     (2)

Where C = annual cost, FC = annual fixed cost, % (interest + taxes + depreciation); this value is of ten rounded off at 16%, P = purchase price, H = hours used per year, R&M= repair and maintenance cost, % per 100 h of use, L= labour cost per year, O = oil cost (often 10% of fuel) per year and F = fuel cost per year.

3.0	RESULT AND DISCUSSION

The summary of the field tests, effects of the tillage tool on some soil physical properties and the operating cost of power tiller are presented in Tables 2-5. The test of significance difference between the measured parameters across the these sites as presented in (Table 3) were not significant for all the parameters consider except for Slippage, which was seen to be significantly higher in Ajase Ipo  with a mean value of about 11.10 % as compare to the mean values of Shaba Maliki and Ejeti (10.53 % each). For the non significant parameters however, it was concluded that the effect of tillage operations is relatively the same for the three sites under study. These imply that the fuel consumption for instance does not differ significantly across the three sites during the experiment. This is also true for all non-significant parameters and also conforms to derivation of basic design of small tractors [2].

Table 4 present the summary of the soil physical properties considered during the experiment. An astute observer will notice that moisture content was seen to record higher mean value after operation than before operation, while cone index was as much as eighty times larger before operation than after operation.

Generally, the following was observed;

- Moisture content and porosity was seen to decrease as the soil depth was increase from

0-7cm through 14 -21cm.

- Bulk density and cone index increase as the soil depth was increase from 0-7cm through 14 -21cm.

This indicates a positive condition for the flow of water and air through the soil profile and minimum resistance to root growth and proliferation. The puddling operation by the power tiller has improved the soil moisture content, reduced shear strength and penetration resistance as proved by [4]. The operating cost of power tiller (Table 5) was determined based on the annual usage and farm size for agricultural purpose. It was evident that the operating cost per annum increases yearly till the year 2005 also the area of coverage and the number of days worked based on 4 hours per day usage. For the first five years of operation in Niger State, the highest area of coverage was recorded in 2005, while the same year recorded highest cost of repair and maintenance and yield (5.2 t/ha). The first three years recorded same volume for repair and maintenance cost until the break down that occurred in 2005 while it was a bit lesser in the subsequent year. The price of diesel varied between N45 per litre to N90 per litre between 2002 and 2006, this also added to the increase in the cost of production yearly. Never the less comparing the income, based on hiring services, there was shortage in 2002. This might be because the operator was yet to be familiar with the machine, he could not make maximum use of the machine to cover much area. For example in 2002, 80 hrs of operation covered 5 hectare while twice the area was covered in 2003 in 120 hrs, the operator was able to save 40 hrs. After 2003 there was a gradual increase in the revenue generation as the hiring rate increases slightly with other factor of production. The machine was not subjected to hiring services for farmers, instead it was given to them freely so that they can be familiar with it and considering the yield at Ajase Ipo within the first two years of power tiller introduction it is hoped that there will be a tremendous increase in subsequent years with proper field management.

4.	CONCLUSIONS

Having determined the impact of using power tiller for sawah for a period of five years at Ejeti and Shaba-Maliki village and two years at Ajase Ipo it is evident that power tiller is the appropriate machinery tool for mechanization of lowland rice production, it is obvious that the power tiller is capable of primary and secondary tillage operations and is most suitable for operations in hilly regions, wet conditions and for small holdings. Given the right set of implements and attachments, the power tiller is capable of performing most of the field operations in the intensive cultivation of sawah rice production. The light weight of power tiller is a favorable factor for working in wet and dry land conditions of sawah fields, it does not sink in wet paddy fields and creates least disturbance to soil structure. In view of the advantages, it has been decided that greater thrust be given to the use of power tillers as it is relatively cheaper than tractors and farmers can afford to invest their money in it. Power tillers are considered appropriate alternative for most farmers in developing countries like Nigeria given their common average size of farm and economic situation. It is also believed that the performance of the power tillers may vary appreciably on the positive side with increasing adoption and mastery of use by owners, operators and mechanics. Mass adoption will also certainly lead to local manufacture of most components giving rise to employment opportunities for many people.

REFERENCES

1Aboaba, F.O., Some aspects of mechanization costings in Nigeria. Proc. Agr. Soc. 				Nigeria (1965) 4:26-30.

2Crosceley, C.P. Theoretical design of small tractors. Paper presented at the Spring 				National Conference of the Institution of Agricultural Engineers, the National 			College of Agricultural Engineering, Silsoe, Bedford, 21 March, 1978. Pp 35-38

3Fashola, O.O., Oladele O.I., Alabi, M.O., Tologbonse, D. and Wakatsuki, T.. Socio-				economics factors influencing the adoption of sawah rice production technology 			in Nigeria. Journal of Food, Agriculture and Environment (JFAE) (2007) (1): 			239-242.

4Fashola O. O., Ademiluyi Y. S., Faleye T., David J.  and T. Wakatsuki. Machinery systems 			management of walking tractor (power tiller) for rice production (sawah) in 			Nigeria. Journal of Food, Agriculture and Environment (JFAE) (2007) (5): 283

5Hunt, D.R., Farm Power and Machinery Management Laboratory Manual and 					Workbook. 7th edn, Iowa State University Press, (1977) p. 365.

6Lambe, T.W., Soil Testing for Engineering. John Wiley and Sons Inc., New York. 1951. p. 33

7WARDA 1988: Strategic plan 1990 – 2000. p. 11.

Table 1: Share of rainfed/irrigated lowland rice areas in Nigeria

| Production system | Major states covered | Estimated share of national rice area (%) | Average yield (t/ha) | Share of rice production (%) |
| --- | --- | --- | --- | --- |
| Rainfed lowland | Akwa Ibom, Bayelsa, Edo Benue, Cross River, Ekiti, Ebonyi, Delta, Ogun, Ondo, Kaduna, Lagos Niger and Rivers states. | 50 | 2.2 | 53 |
| Irrigated | Anambra, Benue, Borno, Cross River, Ebonyi, Enugu, Kano, Kebbi, Kogi, Niger and Sokoto states. | 16 | 3.5 | 27 |

Source: WARDA (1988)

Table 2: Descriptive statistics of field operation result

| Parameters | Site | N | Mean | Std. Deviation | Std. Error |
| --- | --- | --- | --- | --- | --- |
| Slippage | Ejeti | 3 | 10.5300 | .00000 | .00000 |
|  | Shaba Maliki | 3 | 10.5300 | .00000 | .00000 |
|  | Ajase Ipo | 3 | 11.1000 | .00000 | .00000 |
|  | Total | 9 | 10.7200 | .28500 | .09500 |
| Effective field cap. | Ejeti | 3 | .0470 | .00830 | .00479 |
|  | Shaba Maliki | 3 | .0888 | .04619 | .02667 |
|  | Ajase Ipo | 3 | .0551 | .01173 | .00677 |
|  | Total | 9 | .0636 | .03087 | .01029 |
| Theoretical field cap. | Ejeti | 3 | .0504 | .00951 | .00549 |
|  | Shaba Maliki | 3 | .0962 | .04870 | .02812 |
|  | Ajase Ipo | 3 | .0708 | .00888 | .00513 |
|  | Total | 9 | .0725 | .03209 | .01070 |
| Field efficiency | Ejeti | 3 | 93.3733 | 1.16509 | .67266 |
|  | Shaba Maliki | 3 | 91.9600 | 1.52588 | .88097 |
|  | Ajase Ipo | 3 | 91.2633 | .80749 | .46620 |
|  | Total | 9 | 92.1989 | 1.39689 | .46563 |
| Fuel cons.(l/ha) | Ejeti | 3 | 11.1900 | 1.97689 | 1.14136 |
|  | Shaba Maliki | 3 | 12.9133 | .81224 | .46895 |
|  | Ajase Ipo | 3 | 10.5533 | 6.27730 | 3.62420 |
|  | Total | 9 | 11.5522 | 3.48011 | 1.16004 |
| Fuel cons.(l/hr) | Ejeti | 3 | .5370 | .18930 | .10929 |
|  | Shaba Maliki | 3 | 1.1220 | .50554 | .29187 |
|  | Ajase Ipo | 3 | .5370 | .18930 | .10929 |
|  | Total | 9 | .7320 | .40911 | .13637 |
| Area of land (ha) | Ejeti | 3 | .0351 | .00495 | .00286 |
|  | Shaba Maliki | 3 | .0289 | .00656 | .00379 |
|  | Ajase Ipo | 3 | .0351 | .00495 | .00286 |
|  | Total | 9 | .0330 | .00570 | .00190 |
| Average time of operation | Ejeti | 3 | 21.7033 | 3.76513 | 2.17380 |
|  | Shaba Maliki | 3 | 13.1533 | 5.54100 | 3.19910 |
|  | Ajase Ipo | 3 | 18.7033 | 4.53169 | 2.61637 |
|  | Total | 9 | 17.8533 | 5.51968 | 1.83989 |

| Table 3.0: Test of Main Factor Effect (ANOVA) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Sum of Squares | df | Mean Square | F | Sig. |
| Slippage | Between Groups | .650 | 2 | 0.325 | 65535. | 0.001*. |
|  | Within Groups | .000 | 6 | 4.96E-06 |  |  |
|  | Total | .650 | 8 |  |  |  |
| Effective field cap. | Between Groups | .003 | 2 | .001 | 1.888 | .231ns |
|  | Within Groups | .005 | 6 | .001 |  |  |
|  | Total | .008 | 8 |  |  |  |
| Theoretical field cap. | Between Groups | .003 | 2 | .002 | 1.862 | .235ns |
|  | Within Groups | .005 | 6 | .001 |  |  |
|  | Total | .008 | 8 |  |  |  |
| Field efficiency | Between Groups | 6.935 | 2 | 3.467 | 2.398 | .172ns |
|  | Within Groups | 8.676 | 6 | 1.446 |  |  |
|  | Total | 15.610 | 8 |  |  |  |
| Fuel cons.(l/ha) | Between Groups | 8.945 | 2 | 4.472 | .305 | .748ns |
|  | Within Groups | 87.945 | 6 | 14.657 |  |  |
|  | Total | 96.889 | 8 |  |  |  |
| Fuel cons.(l/hr) | Between Groups | .684 | 2 | .342 | 3.137 | .117ns |
|  | Within Groups | .654 | 6 | .109 |  |  |
|  | Total | 1.339 | 8 |  |  |  |
| Area of land (ha) | Between Groups | .000 | 2 | .000 | 1.232 | .356ns |
|  | Within Groups | .000 | 6 | .000 |  |  |
|  | Total | .000 | 8 |  |  |  |
| Average time of operation | Between Groups | 112.905 | 2 | 56.452 | 2.589 | .155ns |
|  | Within Groups | 130.830 | 6 | 21.805 |  |  |
|  | Total | 243.735 | 8 |  |  |  |

*significant at 1% level, ns = not significant

Table 4: Effect of tillage tool on some soil physical properties

| Parameter | Activities | N | Mean | Std. Deviation | Std. Error |
| --- | --- | --- | --- | --- | --- |
| Moisture content | before operation | 3 | 23.6433 | 9.46724 | 5.46591 |
|  | after operation | 3 | 29.5100 | 10.07024 | 5.81405 |
|  | Total | 6 | 26.5767 | 9.31347 | 3.80221 |
| Bulk density | before operation | 3 | 1.5700 | .27875 | .16093 |
|  | after operation | 3 | 1.5200 | .15524 | .08963 |
|  | Total | 6 | 1.5450 | .20364 | .08314 |
| Cone index | before operation | 3 | 51.6800 | 36.45845 | 21.04930 |
|  | after operation | 3 | 6.2033 | 7.94620 | 4.58774 |
|  | Total | 6 | 28.9417 | 34.31301 | 14.00823 |
| Shear strength | before operation | 3 | .0320 | .02458 | .01419 |
|  | after operation | 3 | .0053 | .00503 | .00291 |
|  | Total | 6 | .0187 | .02157 | .00880 |
| Porosity | before operation | 3 | 40.7667 | 10.54008 | 6.08532 |
|  | after operation | 3 | 42.6667 | 5.85861 | 3.38247 |
|  | Total | 6 | 41.7167 | 7.69738 | 3.14244 |

Table 5: Annual use of power tiller

| Year | Total annual use (hr) | Total annual coverage (ha) | R&M cost per annum (N) | Cost of fuel per annum (N) | Total cost of operation per annum (N) | Hiring cost per annum (N) | Yield (t/ha) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2002+ | 80 | 5 | 10, 000 | 2, 722.5 | 27, 554.75 | 15, 000.00 | 5 |
| 2003+ | 120 | 10 | 10, 000 | 6, 050 | 32, 855 | 40, 000.00 | 5.1 |
| 2004+ | 160 | 16 | 10, 000 | 11, 616 | 51, 338 | 80, 000.00 | 5 |
| 2005+ | 220 | 25 | 35, 000 | 25, 606 | 132, 466 | 175, 000.00 | 5.2 |
| 2006+ | 120 | 22 | 32, 444 | 23, 958 | 81, 178 | 154, 000.00 | 5.1 |
| 2009* | 18 | 1 | 5, 000 | 4,806 | 14, 806 | - | 2.4 |
| 2010* | 31 | 2 | 10, 000 | 8, 608 | 18, 608 | - | 4.3 |

+ Usage in Niger State, * Usage in Kwara State